# V5B2 Communication Protocol v1.3

Firmware baseline: 02_V5B2_SENSE_Refact_2ea0133 @ c1e6469

# 1) Common Header

| Field | Type | Bytes | Value/Note |
| --- | --- | --- | --- |
| magic | uint32 | 4 | 0x32423556 ("V5B2") |
| version | uint8 | 1 | 0x01 |
| type | uint8 | 1 | packet type |
| length | uint16 | 2 | payload length |
| seq | uint32 | 4 | sequence |
| timestamp_ms | uint32 | 4 | HAL tick(ms) |

# 2) Packet Types

| Type | Name | Direction | Transport |
| --- | --- | --- | --- |
| 0x01 | HELLO_REQ | PC->FW | TCP |
| 0x02 | HELLO_RESP | FW->PC | TCP |
| 0x10 | TELEMETRY_RAW | FW->PC | UDP |
| 0x11 | TORQUE_CMD | PC->FW | UDP only |
| 0x20/0x21 | TTL_REQ/RESP | Bi-dir | TCP |
| 0x22/0x23/0x24 | SN read/resp/write | Bi-dir | TCP |
| 0x25/0x26 | JAC_REQ/CAL_DONE | Bi-dir | TCP |
| 0x27/0x28 | LED req | PC->FW | TCP |
| 0x29/0x2A/0x2B | NET CFG req/resp/write | Bi-dir | TCP |
| 0x30 | MOTOR_ERROR | FW->PC | TCP |

# 3) Payload Summary

| Payload | Size | Note |
| --- | --- | --- |
| TELEMETRY_RAW | 164B | sample_seq + pos16 + cur16 + sensor16 + vel16 |
| TORQUE_CMD | 32B | int16 torque_mA[16] |
| TTL_RESP | 8B | motor_mask(4) + sensor_mask(4) |
| NET_CONFIG | 14B | ip/mask/gw/tcp_port |
| MOTOR_ERROR | 2B | joint_index,error_status |

# 4) v1.3 Changes (c1e6469)

UDP socket model changed to SINGLE shared socket bound to MY_UDP_PORT(7000).

Telemetry UDP destination port is now learned from TORQUE_CMD source port.

Improved compatibility with PC connected-UDP filters (source port fixed by bind).

UdpTxTask priority raised (AboveNormal) for 3ms cadence stability.